MOELLER MARINE PRODUCTS

A Moore Company

## Product:	Fuel Tanks

# Topic:	Retrofit

# Subject:	Anti-Siphon Valve

## Retrofit Procedures

Supplies Needed:	(1) ¼” x 3/8” anti-siphon valve -15” calibrated (provided).

Tools Needed:	Crescent wrench or 9/16” deep well socket and ratchet.

Retrofit Instructions:

- First check the barb fitting to see if an anti-siphon valve is already in place via visual inspection (see photo 1 and photo 2).  If you are still unable to determine whether an anti-siphon valve is already in place, insert a rod or dowel less than ¼” in diameter into the end of the fitting (see photo 3).  If the rod or dowel penetrates the end of the fitting more than 1 ¾”, then an anti-siphon valve is not present and you should continue.

- Remove clamp and hose from fitting.

- Turning counterclockwise, remove the barb fitting.

- Remove protective cap over threads on the provided anti-siphon valve.

Note: Thread sealant has already been applied to the threads.

- Turning clockwise, hand-tighten the provided anti-siphon valve in order to prevent cross threading.

- Using the crescent wrench or 9/16” deep well socket and ratchet tighten anti-siphon valve.

Note: This is an NPT thread so tighten to a sealed condition.

- Re-install the hose and clamp.

- A pressure check should now be done in accordance with ABYC H-24.17.4.

Do not exceed 3 psi.

- In the event that the replaced fitting demonstrates a leak during the pressure test, tighten the anti-siphon valve until a proper seal is achieved.  Retest.

0.365” Flat

0.565” Flat

* Std Barb fitting (photo 1)		    *Anti-siphon valve fitting (photo 2)	  * Dowel test (photo 3)

10/14/2007