MOELLER MARINE PRODUCTS

A Moore Company

## Product:	Fuel Tanks

# Topic:		Retrofit

# Subject:	Sending Units

## Retrofit Procedures

Supplies Needed:	(5) Washer-Gaskets and (5) Hex-head machine screws. (Provided)

Tools Needed:	Phillips hand screwdriver; Torque wrench which reads in/lb and a 5/16” socket attachment.

Retrofit Instructions:

- Use the “Star Pattern” as illustrated below to remove / replace one screw at a time.

Do not remove the sending unit.

- Replace each removed screw with a washer-gasket and hex-head machine screw combination.  Make sure the gasket part of the washer-gasket is applied to the surface of the sending unit.

- Hand-tighten each replaced screw a few turns; this will prevent cross threading.

- Using the torque wrench tighten the screw to 25 in/lb.

- Continue until all 5 screws are replaced.

Note: If the seal between the sending unit and the fuel tank is broken, the tank should be pressure checked in accordance with ABYC H-24.17.4.  Do not exceed 3 psi.

#1

#2

#3

#4

#5

09/10/2001